Coupling Loss CDF

Root Cause Analysis Report

CBNU vs 3GPP TR 38.901 Calibration Companies

UMa Config1 6 GHz  |  Full Calibration  |  19-Site Wraparound

Chungbuk National University (CBNU)

February 2026

# 1. Executive Summary

CBNU coupling loss CDF is systematically ~10 dB lower (more negative) than other 3GPP calibration companies. Through comprehensive code analysis and comparison with 3GPP TR 38.901 specifications, three root causes were identified that collectively account for the observed offset.

| Root Cause | Location | Excess (dB) | Affected UEs | Avg. Impact |
| --- | --- | --- | --- | --- |
| O2I wall loss fixed at 20 dB (should be ~13.4 dB at 6 GHz) | Link.cpp:1746 | +6.6 dB | 80% (indoor) | ~5.3 dB |
| In-car loss applied to outdoor pedestrian UEs | Link.cpp:1635 | +9.0 dB | 20% (outdoor) | ~1.8 dB |
| UE RSRP divided by P=2 (dual-pol normalization) | Link.cpp:2244 | -3.0 dB | 100% (all) | ~3.0 dB |
| TOTAL ESTIMATED IMPACT |  |  |  | ~10.1 dB |

# 2. Root Cause #1: O2I Building Penetration Loss

## 2.1 Problem Description

For indoor UEs (80% of total) using Channel Model A at carrier frequency f = 6 GHz, the simulator uses a fixed 20 dB wall penetration loss instead of the frequency-dependent formula specified in 3GPP TR 38.901 Table 7.4.3-2.

## 2.2 Code Location

File: src/Link.cpp, Lines 1717-1750

Condition: Channel_Model_Type == 0 (Model A) && carrier_freq <= 6e9

The boundary condition at line 1719 uses carrier_freq > 6000000000. (strictly greater than). Since f = 6 GHz = 6e9 exactly, the code enters the else (below 6 GHz) branch, which uses the simplified model:

Otoi_loss = 20.;

indoor_L = 0.5 * ms[self_ms_idx].ms_d_in;

Otoi_loss = Otoi_loss + indoor_L;   // Missing sigma_P random term!

## 2.3 3GPP Standard (TR 38.901 Table 7.4.3-2)

The correct low-loss O2I model at 6 GHz should use frequency-dependent material losses:

| Parameter | Formula | Value @ 6 GHz |
| --- | --- | --- |
| L_glass (standard multi-pane) | 2 + 0.2 * f_GHz | 3.2 dB |
| L_concrete | 5 + 4 * f_GHz | 29.0 dB |
| PL_tw (wall penetration) | 5 - 10*log10(0.3*10^(-Lg/10) + 0.7*10^(-Lc/10)) | 13.4 dB |
| CBNU implementation | Fixed constant | 20.0 dB |
| EXCESS WALL LOSS |  | +6.6 dB |

Additional issue: The simplified model also omits the stochastic component N(0, sigma_P^2) where sigma_P = 4.4 dB. This reduces the variance of the O2I loss distribution but does not affect the mean.

## 2.4 Recommended Fix

Change the boundary condition at line 1719 from carrier_freq > 6e9 to carrier_freq >= 6e9, so that 6 GHz uses the frequency-dependent model with the proper wall loss formula, indoor distance loss, and sigma_P = 4.4 dB random component.

# 3. Root Cause #2: In-Car Penetration Loss

## 3.1 Problem Description

For outdoor UEs (20% of total), the simulator applies an in-car penetration loss of 9 + 5*N(0,1) dB to all outdoor users. In the 3GPP TR 38.901 Table 7.8-2 calibration specification, outdoor UEs are pedestrian users and should have zero in-car loss.

## 3.2 Code Location

File: src/Link.cpp, Lines 1627-1642

/* Car penetration loss */

if (ms[self_ms_idx].Indoor == true)    // Indoor

incar_loss = 0.;

else if (ms[self_ms_idx].Indoor == false)  // Outdoor

incar_loss = 9. + 5. * randnum.n();  // <-- BUG: applies to ALL outdoor

Note: For the Rural scenario (TYPE == 13, Configuration C), the code correctly distinguishes between pedestrian (60%) and in-car (40%) outdoor users. This distinction is missing for the Dense Urban scenario (TYPE == 12).

## 3.3 Recommended Fix

For 3GPP calibration (Full Calibration mode), set incar_loss = 0 for all outdoor UEs, since calibration assumes pedestrian UEs only. For full system-level simulation, keep the in-car model but make it configurable via cfg parameter.

# 4. Root Cause #3: UE RSRP Normalization

## 4.1 Problem Description

The Get_RSRP function computes antenna gain by summing received power across all UE antenna elements (M=1, N=1, P=2) and then divides by M*N*P = 2. This division effectively halves the computed antenna gain (-3 dB), because it averages over two cross-polarized UE elements rather than summing their contributions to total received power.

## 4.2 Code Location

File: src/Link.cpp, Lines 2183-2246

int M = MS_M;  // = 1

int N = MS_N;  // = 1

int P = MS_P;  // = 2  (cross-polarized)

for (m...) for (n...) for (p...)   // loops over 2 UE elements

alpha += alpha_zero + alpha_nmup;  // accumulates |field|^2

Real RSRP_antenna_gain = alpha / (M*N*P);  // divides by 2 --> loses 3 dB!

## 4.3 Analysis

The TX beamformed field pattern (from Get_BS_antenna_field_pattern) represents one BS polarization. The UE receives this signal via two cross-polarized elements. Each UE element captures one polarization component of the TX field:

- UE pol 0 (p=0): receives F_theta component of TX (co-polarized)

- UE pol 1 (p=1): receives F_phi component of TX (cross-polarized, weak)

The sum alpha = |F_theta|^2 + |F_phi|^2 represents the total field power from one TX polarization. Since the signal formula uses total BS power (49 dBm = both pols combined), dividing alpha by P=2 produces a result that is 3 dB below the correct coupling loss.

## 4.4 Recommended Fix

For the coupling loss computation (and signal/interference when using total BS power), use alpha / (M*N) instead of alpha / (M*N*P). Alternatively, adjust the coupling loss logging to add back 10*log10(P) = 3 dB.

# 5. Verification

## 5.1 Debug CSV Data Confirmation

Analysis of Calib_Debug_Info_drop.csv confirms the relationship: coupling_loss = RSRP - pathloss (verified with < 0.001 dB error for all UEs). The debug data shows:

| Statistic | CBNU Value | Notes |
| --- | --- | --- |
| Mean RSRP | ~2.3 dB | Expected ~5-15 dBi with proper gains |
| Mean Pathloss (final) | ~127.9 dB | Includes O2I + incar + SF |
| Mean Coupling Loss | ~-125.5 dB | Expected ~-115 dB |

## 5.2 Impact Decomposition

The 10 dB offset decomposes as follows:

- O2I excess wall loss: 6.6 dB * 80% indoor UEs = 5.3 dB mean shift

- In-car loss for outdoor: 9.0 dB * 20% outdoor UEs = 1.8 dB mean shift

- RSRP P=2 normalization: 3.0 dB * 100% all UEs = 3.0 dB shift

Total: 5.3 + 1.8 + 3.0 = 10.1 dB (matches observed ~10 dB offset)

# 6. Additional Findings (Minor)

- Missing sigma_P for O2I below 6 GHz: The Gaussian random term N(0, 4.4^2) is omitted from the O2I loss at line 1749. This reduces the variance of the CDF but does not shift the mean.

- cfg parameters verified: All configuration parameters (ISD, BS height, UE height, Tx power, NF, antenna elements, downtilt, carrier frequency) match 3GPP TR 38.901 Table 7.8-2 exactly.

- Path loss formulas verified: UMa LOS and NLOS path loss models in channel.cpp are correctly implemented per TR 38.901 Table 7.4.1-1.

- Beamforming weights verified: DFT beamforming weight normalization (1/sqrt(K*L) per element) is correct. The coherent array gain of 10*log10(K*L) = 12 dB is properly achieved.

- Shadow fading is NOT double-counted: channel.cpp computes sigma_SF but does not add it to pathloss. Link.cpp line 845 correctly adds it once.

# 7. Recommended Fixes (Summary)

| # | File : Line | Current Code | Fix | Impact |
| --- | --- | --- | --- | --- |
| 1 | Link.cpp:1719 | carrier_freq > 6e9 | carrier_freq >= 6e9 | +5.3 dB fix |
| 2 | Link.cpp:1635 | incar_loss = 9+5*N(0,1) | incar_loss = 0 (calibration) | +1.8 dB fix |
| 3 | Link.cpp:2244 | alpha / (M*N*P) | alpha / (M*N) | +3.0 dB fix |