| Radiocommunication Study Groups |  |
| --- | --- |
| Received: Date 20xx Subject: | Document 5D/XXX-E |
|  | XX January 2026 |
|  | English only TECHNOLOGY ASPECTS |
| Korea (Rep. of) |  |
| Proposal for draft new Report (PDNR) ITU-R M.[IMT-2030.EVAL] |  |

1	Introduction

The development of the working document toward the Preliminary Draft New Report (PDNR) ITU-R M.[IMT-2030.EVAL] continued during the 50th meeting of Working Party 5D (WP 5D). Promotion of this document to PDNR status is planned for the 51st meeting. In addition, development of the channel model was initiated at the 50th meeting, with most of its content reused from the channel model specified in ITU-R M.2412.

2	Proposals

The Republic of Korea kindly requests that the proposals below be merged into the working document for further discussion.

Test environments

At the previous WP 5D meeting, seven test environments were agreed, namely: Indoor Hotspot-IC, Dense Urban-IC, Rural-IC, Urban Macro-MC, Indoor Factory-HRLLC, Indoor Factory-ISAC, and Urban Macro-ISAC.

We are of the view that that the total number of test environments should be kept at a reasonable level, taking into account both the necessity of each test environment and the associated evaluation workload. Therefore, it is proposed not to include Sub-Urban Macro-IC and Rural-UC test environments.

Evaluation configurations

Given the diverse frequency ranges and test environments envisioned for IMT-2030, it is essential to define realistic UE antenna configurations that reflect practical device constraints, such as limited physical volume, power consumption, and hardware cost, in order to enable meaningful and realistic performance evaluation.

Modern handheld devices, such as smartphones, integrate multiple radio systems—including cellular, Wi-Fi, Bluetooth, NFC, etc.—within an extremely constrained physical volume. This integration imposes significant limitations on antenna placement, inter-element spacing, and achievable isolation. While non-handheld devices, such as CPEs, offer larger physical volume and fewer industrial-design constraints than smartphones, they are subject to stringent cost requirements in the consumer market. In such cases, even modest increases in the number of antennas can result in substantial increases in RF front-end components and overall system complexity. Therefore, it is necessary to define appropriate antenna configurations that balance practical device constraints with performance expectations across different spectrum ranges.

Accordingly, it is proposed to consider the following UE antenna configurations for the different carrier frequencies and test environments defined in Report ITU-R M.[IMT-2030.EVAL].

|  | 700 MHz | 4 GHz | 7 GHz | 30 GHz |
| --- | --- | --- | --- | --- |
| Number of UE antenna elements | Up to 4 Tx/Rx (For Urban Macro-MC, Up to 2 Tx/Rx) | Up to 4 Tx/Rx | Up to 8 Tx/16Rx | Up to 16 Tx/Rx |

NOTE: Possible device types include handheld device, CPE, vehicle etc. Antenna configurations with a large number of antenna elements may not be applicable to all device types. In particular, configurations assuming 16 Rx antenna elements are intended only for CPE devices. Multi-panel operation is assumed for operation at 30 GHz.

Channel model

Sections 3 and 4 of Annex 1 describe the basic channel model parameters, and the corresponding content from ITU-R M.2412 is expected to remain unchanged in ITU-R M.[IMT-2030.EVAL]. However, one specific paragraph in Section 3 is no longer required, and Step 4 of the large-scale parameter generation process (Section 4.2) requires further clarification and refinement.

Specifically, the paragraph beginning with “The composition of low and high loss…” is unnecessary and is therefore proposed for deletion.

In section 4.2 of ITU-R M.2412, the procedure for generating large-scale parameters refers to the WINNER II channel model. Therefore, in order to make the document self-contained, it is necessary to clearly describe the corresponding procedure. The proposed revision provides the process of generating correlated Gaussian random numbers and includes a detailed explanation of how the generated random variables are mapped to parameters such as DS, K, ASD, and others. In addition, in both the current draft and ITU-R M.2412, parameters such as ASD, ASA, ZSD, and ZSA are mixed with notations such as σ_ASD and σ_ASA, which may cause confusion. To avoid such ambiguity, the standard deviations of the log-normal distributions have already been denoted as σ_lgASD, etc., in the fast-fading parameter table. Accordingly, the notation has been revised and unified by explicitly using the parameter names defined in the tables, such as ASD and lgASD.

The proposed modifications to the channel model included in the working document are provided in Attachment 1.

Attachment:	1

# ATTACHMENT

# Annex 1

Test environments and channel models

[…]

## 3.2	Outdoor to indoor (O-to-I) building penetration loss

[…]

## 3.3	Car penetration loss

[…]

# 4	Fast fading model

[…]

## 4.2	Large scale parameter generation

[…]

Step 4: Generate large scale parameters e.g. root-mean-square delay spread (DS), root-mean-square angular spreads (ASA, ASD, ZSA, ZSD), Ricean K-factor (K) and shadow fading (SF) taking into account cross correlation according to Tables A4-7 to A4-14.

- Construct correlation matrix R whose elements are ρA,B corresponding to the cross-correlation between parameters A and B.

R=1ρK,SFρDS,SFρASD,SFρASA,SFρZSD,SFρZSA,SFρSF,K1ρDS,KρASD,KρASA,KρZSD,KρZSA,KρSF,DSρK,DS1ρASD,DSρASA,DSρZSD,DSρZSA,DSρSF,ASDρK,ASDρDS,ASD1ρASA,ASDρZSD,ASDρZSA,ASDρSF,ASAρK,ASAρDS,ASAρASD,ASA1ρZSD,ASAρZSA,ASAρSF,ZSDρK,ZSDρDS,ZSDρASD,ZSDρASA,ZSD1ρZSA,ZSDρSF,ZSAρK,ZSAρDS,ZSAρASD,ZSAρASA,ZSAρZSD,ZSA1

- Obtain a lower triangular matrix L performing the Cholesky decomposition such that R=L⋅LT.

- Generate a vector of independent standard normal random variables u, and compute the large scale parameter vector:

sM= SF, K, lgDS, lgASD, lgASA, lgZSD, lgZSAT=Σ⋅L⋅u+μ,

where  μ=0, μK, μlgDS, μlgASD, μlgASA, μlgZSD, μlgZSAT and Σ is a diagonal matrix whose diagonal elements are σSF, σK, σlgDS, σlgASD, σlgASA, σlgZSD, and σlgZSA.

- Convert the logarithmic large-scale parameters to linear scale as follows: DS=10lgDS, ASD=10lgASD, ASA=10lgASA, ZSD=10lgZSD, and ZSA=10lgZSA.

These LSPs for different BS-UT links are uncorrelated, but the LSPs for links from co-sited TRxPs to a UT are the same. In addition, these LSPs for the links of UTs on different floors are uncorrelated.

Limit random RMS azimuth arrival and azimuth departure spread values to 104 degrees, i.e. ASA = min(ASA ,104), ASD=min(ASD,104). Limit random RMS zenith arrival and zenith departure spread values to 52 degrees, i.e. ZSA= min(ZSA,52), ZSD= min(ZSD,52).

## 4.3	Small scale parameter generation

[…]

______________

____________________